# Zhone Engineering Academy - Embedded Linux QEMU Labs

Embedded Software Trainee track - Sessions 10 and 11

QEMU ARM Versatile Express for Cortex-A9

Tested with: Ubuntu 24.04.5 LTS, crosstool-NG 1.28.0 (gcc 15.2.0, musl 1.2.5, kernel headers 6.1.147),

U-Boot v2023.01, Linux 6.1.190, BusyBox 1.37.0, Buildroot 2026.02.3, QEMU 8.2.2.

## Prerequisites

Everything below is needed to run all five labs from start to finish on a fresh machine. Prepare it once, before Lab 1.

### Tested environment

| Item | Description |
| --- | --- |
| Operating system | Ubuntu 24.04.5 LTS / 22.04 |
| Host kernel | 7.0.0-38-generic (HWE). No specific host kernel version is required, only the features listed below |
| CPU / RAM | 4 vCPU, 7.7 GB RAM, no swap (the 7.7 GB VM never swapped; the 4 GB minimum of the original lab was not stress-tested) |
| Disk | 59 GB root disk, 50 GB free at the start; the five labs add about 21 GB, so plan for at least 30 GB of free disk space |
| QEMU | 8.2.2 (Debian 1:8.2.2+ds-0ubuntu1.18), from the Ubuntu package, TCG emulation (no KVM needed) |
| Host compiler and tools | gcc 13.3.0, GNU Make 4.3, git 2.43.0, dtc 1.7.0, mksquashfs 4.6.1, GNU ld 2.42, Python 3.12.3, util-linux 2.39.3 (cfdisk, sfdisk), mkfs.fat 4.2 |
| Built by the labs | crosstool-NG 1.28.0 -> arm-linux-gcc 15.2.0 with binutils 2.45, musl 1.2.5 and kernel headers 6.1.147; U-Boot v2023.01; Linux 6.1.190; BusyBox 1.37.0; Buildroot 2026.02.3 |

### Account and permissions

- A regular user that belongs to the sudo group. sudo is used for: apt, losetup, mkfs.vfat, mount/umount, dd to a loop device, running qemu-system-arm with the tap network, editing /etc/exports, exportfs, and copying files into /srv/tftp. No other group membership is needed (no kvm group).

- Before Lab 4 you must set a git identity (needed by git am):

$ git config --global user.name "Your Name"

$ git config --global user.email you@example.com

### Packages (complete list)

$ sudo apt update

$ sudo apt dist-upgrade

$ sudo apt install build-essential git autoconf bison flex texinfo help2man gawk libtool-bin \

libncurses5-dev unzip gettext python3 rsync \

qemu-user qemu-system-arm \

libssl-dev device-tree-compiler swig python3-dev python3-setuptools \

tftpd-hpa bc nfs-kernel-server squashfs-tools \

dosfstools fdisk wget cpio file

### Host kernel features and devices

- loop block devices with partition scanning (losetup --partscan produced loop10: p1 p2 p3 on the test machine), and at least one free loop device.

- /dev/net/tun must exist (it was crw-rw-rw- 1 root root 10, 200); QEMU's -net tap needs it. Running QEMU with the tap network needs root, hence sudo qemu-system-arm.

- The NFS server kernel modules (nfsd, sunrpc, lockd, ...) are loaded by the nfs-kernel-server package.

- SquashFS support is only needed in the target kernel, not on the host.

### Services and configuration files

TFTP (tftpd-hpa): the service starts automatically on installation and the default configuration needs no change. /etc/default/tftpd-hpa:

# /etc/default/tftpd-hpa

TFTP_USERNAME="tftp"

TFTP_DIRECTORY="/srv/tftp"

TFTP_ADDRESS=":69"

TFTP_OPTIONS="--secure"

/srv/tftp is owned by root:nogroup (drwxr-xr-x), so every file you publish over TFTP (text file in Lab 2, zImage and .dtb in Labs 3 and 5) must be copied with sudo cp. (Alternatively run sudo chown $USER /srv/tftp once.)

NFS (nfs-kernel-server): the nfs-server and rpcbind services start automatically on installation. Lab 4 and Lab 5 add one export line each to /etc/exports, restricted to the target address 192.168.0.100:

/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot 192.168.0.100(rw,no_root_squash,no_subtree_check)

/home/longnguyen/embedded-linux-qemu-labs/buildroot/nfsroot 192.168.0.100(rw,no_root_squash,no_subtree_check)

followed by sudo exportfs -r. sudo exportfs -v then shows 192.168.0.100(sync,wdelay,hide,no_subtree_check,sec=sys,rw,secure,no_root_squash,no_all_squash) for each path. The target mounts the export with NFS version 3 over TCP. The home directory (mode 750 on Ubuntu) does not block the NFS server, which runs as root.

qemu-myifup: a small script created in Lab 2 next to sd.img (content given there).

### Network and firewall

- Host and target talk over a private link created by QEMU (tap0): host 192.168.0.1, target 192.168.0.100, network 192.168.0.0/24. This range must not clash with the network your host is already connected to (use another range and adapt every address in Labs 2 to 5 if it does). tap0 is created by QEMU for each run and disappears when QEMU exits.

- Firewall: on the test machine ufw was inactive and the iptables policies were ACCEPT, so TFTP (UDP 69) and NFS (TCP 2049 and 111) were reachable on tap0. If ufw is active on your machine, you must allow traffic on the tap0 interface; this case was not tested.

### Environment variables (per shell)

These variables are not persistent: set them again in every new terminal that needs them.

$ export PATH=$HOME/x-tools/arm-training-linux-musleabihf/bin:$PATH    # Labs 1 to 4 (cross-compiler)

$ export ARCH=arm                                                      # kernel only (Lab 3)

$ export CROSS_COMPILE=arm-linux-                                      # U-Boot, kernel, BusyBox (also for BusyBox "make install")

Run Buildroot's make (Lab 5) from a shell where ARCH and CROSS_COMPILE are not exported; the tested run used such a shell.

## Training setup

### Download files and directories used in practical labs

Download and extract its tarball from a terminal:

$ cd

$ wget https://bootlin.com/doc/training/embedded-linux/embedded-linux-qemu-labs.tar.xz

$ tar xvf embedded-linux-qemu-labs.tar.xz

Lab data are now available in the embedded-linux-qemu-labs directory in your home directory. This directory is also your working space: each lab uses its own subdirectory (toolchain, bootloader, kernel, tinysystem, buildroot).

### Update your distribution

$ sudo apt update

$ sudo apt dist-upgrade

## Lab 1 - Building a cross-compiling toolchain (Session 10)

Objective: Learn how to build your own cross-compiling toolchain for the musl C library.

After this lab, you will be able to:

- Configure the Crosstool-NG tool

- Execute Crosstool-NG and build your own cross-compiling toolchain

- Check that the toolchain produces ARM binaries and inspect what it contains

### Setup

Go to the $HOME/embedded-linux-qemu-labs/toolchain directory. This lab needs a system with at least 4 GB of RAM (about 12 GB of free disk space for the build, see the Prerequisites).

Install the packages needed for this lab:

$ sudo apt install build-essential git autoconf bison flex texinfo help2man gawk libtool-bin \

libncurses5-dev unzip gettext python3 rsync

### Getting Crosstool-NG

Download the Crosstool-NG sources from git and switch to the tested release:

$ cd ~/embedded-linux-qemu-labs/toolchain

$ git clone https://github.com/crosstool-ng/crosstool-ng

$ cd crosstool-ng/

$ git checkout crosstool-ng-1.28.0

### Building and installing Crosstool-NG

Because we build from a git repository rather than a release archive, generate the configure script first. We keep Crosstool-NG local to its source directory instead of installing it system-wide:

$ ./bootstrap

$ ./configure --enable-local

$ make

Check that it works with ./ct-ng help.

### Configure the toolchain to produce

One Crosstool-NG installation can produce any number of toolchains (different architectures, C libraries and component versions). It comes with ready-made configurations called samples: list them with ./ct-ng list-samples.

The only Cortex-A9 sample is arm-cortexa9_neon-linux-gnueabihf (shown as [L..X] in the list; the X means EXPERIMENTAL). Load it:

$ ./ct-ng list-samples

$ ./ct-ng arm-cortexa9_neon-linux-gnueabihf

The sample already selects gcc 15.2.0, C++ support and Try features marked as EXPERIMENTAL. It also ships glibc, Linux 6.16 kernel headers, GDB and the vendor string cortexa9_neon, which you change in the next step.

Refine the configuration with the menu interface:

$ ./ct-ng menuconfig

Set the following options (the symbol written to .config is given in parentheses):

- Path and misc options: make sure Try features marked as EXPERIMENTAL is enabled (CT_EXPERIMENTAL=y, already set by the sample). Extra build compiler flags (CT_EXTRA_CXXFLAGS_FOR_BUILD) was left empty: the -fno-char8_t flag was not needed with the host gcc 13.3 used for the test (it was not tested with newer host compilers).

- Toolchain options: set Tuple's vendor string (CT_TARGET_VENDOR) to training and Tuple's alias (CT_TARGET_ALIAS) to arm-linux. The compiler can then be called as arm-linux-gcc.

- Operating System: set Version of linux to 6.1 (CT_LINUX_V_6_1=y, resolving to CT_LINUX_VERSION="6.1.147"). The kernel headers inside the toolchain must not be newer than the kernel that will run on the target (6.1.190 in these labs).

- C-library: set the C library to musl (CT_LIBC_MUSL=y, CT_LIBC_GLIBC unset).

- C compiler: keep Version of gcc at 15.2.0 (CT_GCC_VERSION="15.2.0") and make sure C++ (CT_CC_LANG_CXX=y) is enabled; both are already set by the sample.

- Debug facilities: remove all options (the sample enables GDB: unset CT_DEBUG_GDB). Debugging tools can come from the filesystem build tools instead.

Browse the other menus and read the help of some options, but do not customize further: the labs were tested with the configuration above.

The same result can be obtained without the menu interface, by editing .config and letting Crosstool-NG resolve the dependencies. This is the exact sequence that was run in the test, after loading the sample:

$ sed -i -e 's/^CT_LINUX_V_6_16=y/# CT_LINUX_V_6_16 is not set/' \

-e 's/^# CT_LINUX_V_6_1 is not set/CT_LINUX_V_6_1=y/' \

-e 's/^CT_LIBC_GLIBC=y/# CT_LIBC_GLIBC is not set\nCT_LIBC_MUSL=y/' \

-e 's/^CT_TARGET_VENDOR=.*/CT_TARGET_VENDOR="training"/' \

-e 's/^CT_TARGET_ALIAS=.*/CT_TARGET_ALIAS="arm-linux"/' \

-e 's/^CT_DEBUG_GDB=y/# CT_DEBUG_GDB is not set/' .config

$ ./ct-ng olddefconfig

Check the resulting target tuple:

$ ./ct-ng show-tuple

Expected output:

arm-training-linux-musleabihf

Key lines of the resulting .config:

CT_EXPERIMENTAL=y

CT_LOCAL_TARBALLS_DIR="${HOME}/src"

CT_PREFIX_DIR="${CT_PREFIX:-${HOME}/x-tools}/${CT_HOST:+HOST-${CT_HOST}/}${CT_TARGET}"

CT_PARALLEL_JOBS=0

CT_ARCH_CPU="cortex-a9"

CT_ARCH_FPU="neon"

CT_ARCH_FLOAT_HW=y

CT_ARCH_FLOAT="hard"

CT_TARGET_VENDOR="training"

CT_TARGET_ALIAS="arm-linux"

CT_LINUX_V_6_1=y

CT_LINUX_VERSION="6.1.147"

CT_BINUTILS_VERSION="2.45"

CT_LIBC_MUSL=y

CT_MUSL_VERSION="1.2.5"

CT_GCC_VERSION="15.2.0"

CT_CC_LANG_CXX=y

No CT_DEBUG_*=y line is present.

### Produce the toolchain

$ ./ct-ng build

The toolchain is installed in $HOME/x-tools/ by default. Wait for the build to finish (27 minutes on the 4 vCPU test machine). The build prints its progress; the beginning and the end of the log of the test run:

[WARN ]  Number of open files 1024 may not be sufficient to build the toolchain; increasing to 2048

[INFO ]  Build started 20261020.170948

[WARN ]  Directory '/home/longnguyen/src' does not exist.

[WARN ]  Will not save downloaded tarballs to local storage.

[INFO ]  Retrieving needed toolchain components' tarballs: done in 201.82s (at 03:24)

[INFO ]  Extracting and patching toolchain components: done in 36.54s (at 04:01)

[INFO ]  Installing ncurses for build: done in 24.32s (at 04:25)

[INFO ]  Installing GMP for host: done in 33.84s (at 05:10)

[INFO ]  Installing MPFR for host: done in 22.16s (at 05:32)

[INFO ]  Installing ISL for host: done in 29.01s (at 06:01)

[INFO ]  Installing binutils for host: done in 74.89s (at 07:50)

[INFO ]  Installing kernel headers: done in 14.69s (at 08:05)

[INFO ]  Installing core C gcc compiler: done in 506.89s (at 16:32)

[INFO ]  Installing C library: done in 28.37s (at 17:00)

[INFO ]  Installing final gcc compiler: done in 608.28s (at 27:08)

[INFO ]  Finalizing the toolchain's directory: done in 5.02s (at 27:13)

[INFO ]  Build completed at 20261020.173701

[INFO ]  (elapsed: 27:12.41)

The two [WARN ] lines about ~/src only mean that downloaded tarballs are not kept. After the build, .build takes 12 GB and ~/x-tools 332 MB.

### Testing the toolchain

Add $HOME/x-tools/arm-training-linux-musleabihf/bin/ to your PATH, then compile the hello.c program from your lab directory:

$ export PATH=$HOME/x-tools/arm-training-linux-musleabihf/bin:$PATH

$ cd ~/embedded-linux-qemu-labs/toolchain

$ arm-linux-gcc -o hello hello.c

Use file hello to confirm the binary is built for ARM:

$ file hello

Expected output:

hello: ELF 32-bit LSB executable, ARM, EABI5 version 1 (SYSV), dynamically linked, interpreter /lib/ld-musl-armhf.so.1, not stripped

You can still run this binary on your x86 host with the QEMU user emulator, which emulates only the instruction set, not a whole machine:

$ sudo apt install qemu-user

$ qemu-arm hello

Expected output:

qemu-arm: Could not open '/lib/ld-musl-armhf.so.1': No such file or directory

The emulator is missing the ARM dynamic loader that the binary requires. Find it in the toolchain:

$ find ~/x-tools -name ld-musl-armhf.so.1

Expected output:

/home/longnguyen/x-tools/arm-training-linux-musleabihf/arm-training-linux-musleabihf/sysroot/lib/ld-musl-armhf.so.1

Tell qemu-arm where the target libraries are with -L:

$ qemu-arm -L ~/x-tools/arm-training-linux-musleabihf/arm-training-linux-musleabihf/sysroot hello

Expected output:

Hello world!

### Exploring the toolchain

Browse the toolchain directory and identify the binutils tools (arm-linux-as, arm-linux-ld, arm-linux-readelf, ...), the compiler, the C library and the kernel headers inside the sysroot. Every tool is available under two names: the alias prefix arm-linux- and the full tuple prefix arm-training-linux-musleabihf-.

$ arm-linux-gcc --version

Expected output:

arm-linux-gcc (crosstool-NG 1.28.0) 15.2.0

Copyright (C) 2025 Free Software Foundation, Inc.

This is free software; see the source for copying conditions.  There is NO

warranty; not even for MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.

Use arm-linux-gcc -v to see how the compiler was configured:

$ arm-linux-gcc -v

Expected output (the long Configured with: line is shortened):

Using built-in specs.

COLLECT_GCC=arm-linux-gcc

COLLECT_LTO_WRAPPER=/home/longnguyen/x-tools/arm-training-linux-musleabihf/libexec/gcc/arm-training-linux-musleabihf/15.2.0/lto-wrapper

Target: arm-training-linux-musleabihf

Configured with: /home/longnguyen/embedded-linux-qemu-labs/toolchain/crosstool-ng/.build/arm-training-linux-musleabihf/src/gcc/configure --build=x86_64-build_pc-linux-gnu --host=x86_64-build_pc-linux-gnu --target=arm-training-linux-musleabihf [...] --enable-languages=c,c++ --with-cpu=cortex-a9 --with-fpu=neon --with-float=hard --with-pkgversion='crosstool-NG 1.28.0' [...] --disable-multilib [...]

Thread model: posix

Supported LTO compression algorithms: zlib zstd

gcc version 15.2.0 (crosstool-NG 1.28.0)

List the tools installed in the toolchain (start of the listing):

$ ls ~/x-tools/arm-training-linux-musleabihf/bin

Expected output (start):

arm-linux-addr2line

arm-linux-ar

arm-linux-as

arm-linux-c++

arm-linux-cc

arm-linux-c++filt

arm-linux-cpp

arm-linux-elfedit

arm-linux-g++

arm-linux-gcc

arm-linux-gcc-15.2.0

arm-linux-gcc-ar

[...]

arm-linux-ld

arm-linux-ld.bfd

arm-linux-ldd

[...]

arm-linux-nm

arm-linux-objcopy

arm-linux-objdump

[...]

arm-linux-readelf

arm-linux-size

arm-linux-strings

arm-linux-strip

arm-training-linux-musleabihf-addr2line

[...]

Check which kernel headers were installed in the sysroot:

$ grep LINUX_VERSION_CODE ~/x-tools/arm-training-linux-musleabihf/arm-training-linux-musleabihf/sysroot/usr/include/linux/version.h

Expected output:

#define LINUX_VERSION_CODE 393619

393619 is 0x060193, that is kernel 6.1.147: not newer than the 6.1.190 kernel of Lab 3.

## Lab 2 - Bootloader - U-Boot (Session 11)

Objectives: Compile and install the U-Boot bootloader, use basic U-Boot commands, and set up TFTP communication with the development workstation.

### Setup

Go to the $HOME/embedded-linux-qemu-labs/bootloader directory. Install the packages for this lab, including qemu-system-arm. In this and the following labs, a QEMU-emulated ARM Versatile Express Cortex-A9 board is used.

$ sudo apt install qemu-system-arm libssl-dev device-tree-compiler swig python3-dev python3-setuptools

Make the Lab 1 toolchain available in your PATH:

$ export PATH=$HOME/x-tools/arm-training-linux-musleabihf/bin:$PATH

### Configuring and building U-Boot

$ cd ~/embedded-linux-qemu-labs/bootloader

$ git clone https://git.u-boot-project.org/u-boot/u-boot.git

$ cd u-boot

$ git checkout v2023.01

Configure U-Boot for the Versatile Express Cortex-A9 board (vexpress_ca9x4_defconfig). Read the Building the Software section of the README to understand the configuration and build steps. You must give the cross-compiler prefix (the part before gcc):

$ export CROSS_COMPILE=arm-linux-

$ make vexpress_ca9x4_defconfig

Then run make menuconfig:

- In the Environment submenu, make U-Boot store its environment in a file called uboot.env on a FAT filesystem on an MMC/SD card (the emulated machine has no flash): unset Environment in flash memory (CONFIG_ENV_IS_IN_FLASH), set Environment is in a FAT filesystem (CONFIG_ENV_IS_IN_FAT), set the block device name (CONFIG_ENV_FAT_INTERFACE) to mmc, and set the device and partition (CONFIG_ENV_FAT_DEVICE_AND_PART) to 0:1.

- Add support for the editenv (CONFIG_CMD_EDITENV) and bootd (CONFIG_CMD_BOOTD) commands.

The same configuration can be applied without the menu interface; this is the equivalent sequence that was run in the test:

$ scripts/config -d ENV_IS_IN_FLASH -e ENV_IS_IN_FAT --set-str ENV_FAT_INTERFACE mmc \

--set-str ENV_FAT_DEVICE_AND_PART 0:1 -e CMD_EDITENV -e CMD_BOOTD

$ make olddefconfig

Resulting lines in u-boot/.config (before the change the defconfig had CONFIG_ENV_IS_IN_FLASH=y, and CONFIG_CMD_EDITENV and CONFIG_CMD_BOOTD were unset):

CONFIG_ENV_SIZE=0x40000

CONFIG_CMD_BOOTD=y

CONFIG_CMD_EDITENV=y

CONFIG_ENV_OVERWRITE=y

CONFIG_ENV_IS_IN_FAT=y

CONFIG_ENV_FAT_INTERFACE="mmc"

CONFIG_ENV_FAT_DEVICE_AND_PART="0:1"

CONFIG_ENV_FAT_FILE="uboot.env"

CONFIG_SYS_MMC_ENV_DEV=0

CONFIG_FS_FAT=y

CONFIG_FAT_WRITE=y

Build it (use make -j<n> to go faster; the test used -j4 and the build took 17 seconds):

$ make -j4

U-Boot v2023.01 builds with the Lab 1 gcc 15.2.0 toolchain without any change. The build prints a few notices about CONFIG_OF_EMBED, CONFIG_TIMER and CONFIG_DM_SERIAL (board-removal deadline notices for this old release); they are warnings, not errors. The build produces several binaries, including u-boot (4577472 bytes) and u-boot.bin (567304 bytes).

### Testing U-Boot

Go back to the bootloader directory and start QEMU with U-Boot:

$ cd ~/embedded-linux-qemu-labs/bootloader

$ qemu-system-arm -M vexpress-a9 -m 128M -nographic -kernel u-boot/u-boot

- -M: the emulated machine

- -m: the amount of memory

- -nographic: use the terminal as the serial console (the equivalent of the USB-serial adapter on a real board)

- -kernel: load the binary straight into the emulated machine and run it, so no first-stage bootloader is needed (this is not possible on real hardware)

If your machine has no sound device, QEMU prints many alsa: Could not initialize DAC ... and audio: Could not create a backend for voice 'lm4549.out' lines. They are harmless; adding -audio none to the QEMU command line removes them. The captures in this guide have these lines removed.

Press a key before the timeout ends to reach the U-Boot prompt, and try help, version and bdinfo. To exit QEMU, press Ctrl-a then x (Ctrl-a then h lists the other commands).

Expected output (no SD card attached yet, so U-Boot cannot read its environment; the help list is shortened):

U-Boot 2023.01 (Oct 20 2026 - 17:37:55 +0000)

DRAM:  128 MiB

WARNING: Caches not enabled

Core:  18 devices, 10 uclasses, devicetree: embed

Flash: 64 MiB

MMC:   mmci@5000: 0

Loading Environment from FAT... Card did not respond to voltage select! : -110

** Bad device specification mmc 0 **

In:    serial

Out:   serial

Err:   serial

Net:   eth0: ethernet@3,02000000

Hit any key to stop autoboot:  2  0

=> help

?         - alias for 'help'

base      - print or set address offset

bdinfo    - print Board Info structure

blkcache  - block cache diagnostics and control

boot      - boot default, i.e., run 'bootcmd'

bootd     - boot default, i.e., run 'bootcmd'

[...]

tftpboot  - load file via network using TFTP protocol

true      - do nothing, successfully

ubi       - ubi commands

ubifsload - load file from an UBIFS filesystem

ubifsls   - list files in a directory

ubifsmount- mount UBIFS volume

ubifsumount- unmount UBIFS volume

version   - print monitor, compiler and linker version

=> version

U-Boot 2023.01 (Oct 20 2026 - 17:37:55 +0000)

arm-linux-gcc (crosstool-NG 1.28.0) 15.2.0

GNU ld (crosstool-NG 1.28.0) 2.45

=> bdinfo

boot_params = 0x60002000

DRAM bank   = 0x00000000

-> start    = 0x60000000

-> size     = 0x08000000

DRAM bank   = 0x00000001

-> start    = 0x80000000

-> size     = 0x00000004

flashstart  = 0x40000000

flashsize   = 0x04000000

flashoffset = 0x00000000

baudrate    = 38400 bps

relocaddr   = 0x67f68000

reloc off   = 0x07768000

Build       = 32-bit

current eth = ethernet@3,02000000

ethaddr     = 52:54:00:12:34:56

IP addr     = <NULL>

fdt_blob    = 0x608781e0

[...]

The Card did not respond to voltage select! : -110 message is expected at this stage: no SD image is attached. The bdinfo output shows that RAM starts at 0x60000000 and is 0x08000000 bytes (128 MiB) long. The second bank (0x80000000, 4 bytes) is a quirk of this emulated board; ignore it.

### SD card setup

Go back to the bootloader directory. U-Boot's environment needs somewhere to be stored, and later labs will use the same storage for other purposes. Create a 1 GB image filled with zeros:

$ dd if=/dev/zero of=sd.img bs=1M count=1024

Expected output:

1024+0 records in

1024+0 records out

1073741824 bytes (1.1 GB, 1.0 GiB) copied, 0.43753 s, 2.5 GB/s

Partition the image with three primary partitions, from the beginning:

- 64 MB, FAT16 type, marked bootable

- 8 MB, Linux type

- The rest of the image, Linux type

cfdisk sd.img (choose the dos label type) is the interactive way to do it. The non-interactive command below creates the same layout and is the one that was run in the test:

$ printf 'label: dos\n,64M,6,*\n,8M,L\n,,L\n' | sfdisk sd.img

Expected output:

Checking that no-one is using this disk right now ... OK

Disk sd.img: 1 GiB, 1073741824 bytes, 2097152 sectors

Units: sectors of 1 * 512 = 512 bytes

Sector size (logical/physical): 512 bytes / 512 bytes

I/O size (minimum/optimal): 512 bytes / 512 bytes

>>> Script header accepted.

>>> Created a new DOS (MBR) disklabel with disk identifier 0xce9f4869.

sd.img1: Created a new partition 1 of type 'FAT16' and of size 64 MiB.

sd.img2: Created a new partition 2 of type 'Linux' and of size 8 MiB.

sd.img3: Created a new partition 3 of type 'Linux' and of size 951 MiB.

sd.img4: Done.

New situation:

Disklabel type: dos

Disk identifier: 0xce9f4869

Device     Boot  Start     End Sectors  Size Id Type

sd.img1    *      2048  133119  131072   64M  6 FAT16

sd.img2         133120  149503   16384    8M 83 Linux

sd.img3         149504 2097151 1947648  951M 83 Linux

The partition table has been altered.

Syncing disks.

Then use the loop driver to expose the image and its partitions as block devices:

$ sudo losetup -f --show --partscan sd.img

Expected output (the loop device number may differ on your machine; the following steps use /dev/loop<x>):

/dev/loop10

Check with sudo dmesg | tail that three partitions were detected (the other lines of the output are unrelated kernel messages):

[...]

[ 2202.941236] loop10: detected capacity change from 0 to 2097152

[ 2202.942887]  loop10: p1 p2 p3

Format the first partition as FAT16 with the label boot, then release the loop device:

$ sudo mkfs.vfat -F 16 -n boot /dev/loop<x>p1

$ sudo losetup -d /dev/loop<x>

Expected output of mkfs.vfat:

mkfs.fat: Warning: lowercase labels might not work properly on some systems

mkfs.fat 4.2 (2021-01-31)

The warning about the lowercase label is harmless.

### Testing U-Boot's environment

Start QEMU again with the SD image:

$ qemu-system-arm -M vexpress-a9 -m 128M -nographic -kernel u-boot/u-boot -sd sd.img

QEMU 8.2.2 prints a warning that the format of sd.img was not specified and that raw was guessed; it is harmless. On this first boot U-Boot finds no uboot.env yet. At the U-Boot prompt, store a variable, check that the file was created on the FAT partition, reboot, and check the variable persisted:

=> setenv foo bar

=> saveenv

=> fatls mmc 0:1

=> reset

=> printenv foo

Expected output:

WARNING: Image format was not specified for 'sd.img' and probing guessed raw.

Automatically detecting the format is dangerous for raw images, write operations on block 0 will be restricted.

Specify the 'raw' format explicitly to remove the restrictions.

U-Boot 2023.01 (Oct 20 2026 - 17:37:55 +0000)

DRAM:  128 MiB

WARNING: Caches not enabled

Core:  18 devices, 10 uclasses, devicetree: embed

Flash: 64 MiB

MMC:   mmci@5000: 0

Loading Environment from FAT... Unable to read "uboot.env" from mmc0:1...

In:    serial

Out:   serial

Err:   serial

Net:   eth0: ethernet@3,02000000

Hit any key to stop autoboot:  2  0

=> setenv foo bar

=> saveenv

Saving Environment to FAT... OK

=> fatls mmc 0:1

262144   uboot.env

1 file(s), 0 dir(s)

=> reset

resetting ...

U-Boot 2023.01 (Oct 20 2026 - 17:37:55 +0000)

DRAM:  128 MiB

WARNING: Caches not enabled

Core:  18 devices, 10 uclasses, devicetree: embed

Flash: 64 MiB

MMC:   mmci@5000: 0

Loading Environment from FAT... OK

In:    serial

Out:   serial

Err:   serial

Net:   eth0: ethernet@3,02000000

Hit any key to stop autoboot:  2  0

=> printenv foo

foo=bar

The first boot message Unable to read "uboot.env" from mmc0:1... is expected; after saveenv the environment is loaded with Loading Environment from FAT... OK.

### Setup networking between QEMU and the host

Create a script qemu-myifup in the bootloader directory that brings up the host side of the link:

#!/bin/sh

/sbin/ip a add 192.168.0.1/24 dev $1

/sbin/ip link set $1 up

Make it executable with chmod +x qemu-myifup. The host uses 192.168.0.1 and the target uses 192.168.0.100. Use another address range if it collides with your local network. Start QEMU with root privileges and a tap interface:

$ sudo qemu-system-arm -M vexpress-a9 -m 128M -nographic \

-kernel u-boot/u-boot -sd sd.img \

-net tap,script=./qemu-myifup -net nic

- -net tap: creates a software network interface on the host

- -net nic: adds a network device to the emulated machine

On the host (in another terminal, while QEMU is running), check that tap0 exists with the expected address:

$ ip a show tap0

Expected output (the MAC and link-local addresses differ on your machine):

3: tap0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc fq_codel state UNKNOWN group default qlen 1000

link/ether c2:49:c5:42:52:9c brd ff:ff:ff:ff:ff:ff

inet 192.168.0.1/24 scope global tap0

valid_lft forever preferred_lft forever

inet6 fe80::c049:c5ff:fe42:529c/64 scope link tentative

valid_lft forever preferred_lft forever

In U-Boot, configure and save the network settings, then test the link:

=> setenv ipaddr 192.168.0.100

=> setenv serverip 192.168.0.1

=> saveenv

=> ping 192.168.0.1

Expected output:

=> setenv ipaddr 192.168.0.100

=> setenv serverip 192.168.0.1

=> saveenv

Saving Environment to FAT... OK

=> ping 192.168.0.1

smc911x: detected LAN9118 controller

smc911x: phy initialized

smc911x: MAC 52:54:00:12:34:56

Using ethernet@3,02000000 device

smc911x: MAC 52:54:00:12:34:56

host 192.168.0.1 is alive

It must end with host 192.168.0.1 is alive.

### Setting up the TFTP server

$ sudo apt install tftpd-hpa

The service starts automatically and its default configuration (see the Prerequisites) is enough. In U-Boot, run bdinfo (done above): RAM starts at 0x60000000, so use 0x61000000 to test TFTP.

Put a small text file in the directory exported by the TFTP server. The directory is owned by root, so use sudo:

$ echo 'Hello from the TFTP server' > /tmp/textfile.txt

$ sudo cp /tmp/textfile.txt /srv/tftp/

Then in U-Boot:

=> tftp 0x61000000 textfile.txt

=> md 0x61000000

md dumps memory so you can verify the download. Expected output:

=> tftp 0x61000000 textfile.txt

smc911x: detected LAN9118 controller

smc911x: phy initialized

smc911x: MAC 52:54:00:12:34:56

Using ethernet@3,02000000 device

TFTP from server 192.168.0.1; our IP address is 192.168.0.100

Filename 'textfile.txt'.

Load address: 0x61000000

Loading: *#

4.9 KiB/s

done

Bytes transferred = 27 (1b hex)

smc911x: MAC 52:54:00:12:34:56

=> md 0x61000000

61000000: 6c6c6548 7266206f 74206d6f 54206568  Hello from the T

61000010: 20505446 76726573 000a7265 00000000  FTP server......

61000020: 00000000 00000000 00000000 00000000  ................

[...]

610000f0: 00000000 00000000 00000000 00000000  ................

You can also display the whole environment with printenv to see that ipaddr and serverip are saved:

=> printenv

arch=arm

baudrate=38400

board=vexpress

board_name=vexpress

[...]

bootargs=console=tty0 console=ttyAMA0,38400n8

bootcmd=run distro_bootcmd; run bootflash

[...]

ipaddr=192.168.0.100

[...]

serverip=192.168.0.1

stderr=serial

stdin=serial

stdout=serial

[...]

vendor=armltd

Environment size: 4336/262140 bytes

### Rescue binary

If you cannot produce a working bootloader, or later lose yours, a working version is provided as data/u-boot in the bootloader lab directory (it was present in the lab data of the test run but not used).

## Lab 3 - Kernel - fetching sources, cross-compiling and booting (Session 11)

Objective: Fetch the Linux sources, cross-compile a kernel for the emulated ARM machine, and boot it from U-Boot over TFTP.

After this lab, you will be able to:

- Get the sources of a stable Linux kernel release from git

- Set up a cross-compiling environment and cross-compile the kernel

- Use U-Boot to download the kernel and the device tree

- Check that the kernel starts and automate the boot

- Decompile a device tree blob and read what it describes

### Setup

Create and enter $HOME/embedded-linux-qemu-labs/kernel. Start the download early (before the end of Lab 2): even without the full history it is a sizeable download (1 min 42 s for 1.7 GB on the test machine).

$ mkdir -p ~/embedded-linux-qemu-labs/kernel

$ cd ~/embedded-linux-qemu-labs/kernel

### Fetching the sources

These labs use the 6.1 stable (long-term support) series. Clone only that branch from the stable tree, without the full history, to keep the download small:

$ git clone --depth 1 --branch linux-6.1.y https://git.kernel.org/pub/scm/linux/kernel/git/stable/linux.git

$ cd linux

$ make kernelversion

Expected output of make kernelversion (a 6.1.x version; 6.1.190 at the time of the test):

6.1.190

### Cross-compiling environment

$ export PATH=$HOME/x-tools/arm-training-linux-musleabihf/bin:$PATH

$ export ARCH=arm

$ export CROSS_COMPILE=arm-linux-

$ sudo apt install bc

### Kernel configuration

Use make help to find the default configuration target for the Versatile Express boards and apply it:

$ make help | grep -i vexpress

$ make vexpress_defconfig

Expected output of the first command:

vexpress_defconfig          - Build for vexpress

vexpress_defconfig already enables everything the next labs need: NFS root (CONFIG_ROOT_NFS, CONFIG_NFS_FS, CONFIG_NFS_V3), IP autoconfiguration (CONFIG_IP_PNP), SquashFS (CONFIG_SQUASHFS with zlib and LZO decompressors, no extended attributes), CONFIG_DEVTMPFS, the SMSC911x network driver, the MMC driver and VFAT. Only two changes are needed. Run make menuconfig:

- Disable CONFIG_GCC_PLUGINS (it is enabled by the defconfig and needs extra dependencies).

- Add CONFIG_DEVTMPFS_MOUNT (the kernel then mounts devtmpfs on /dev itself at boot).

The equivalent command line, as run in the test:

$ scripts/config -d GCC_PLUGINS -e DEVTMPFS_MOUNT

$ make olddefconfig

Relevant lines of the resulting .config:

CONFIG_VFP=y  CONFIG_NEON=y  CONFIG_SMP=y

CONFIG_HAVE_GCC_PLUGINS=y

# CONFIG_GCC_PLUGINS is not set

CONFIG_DEVTMPFS=y

CONFIG_DEVTMPFS_MOUNT=y

CONFIG_IP_PNP=y  CONFIG_IP_PNP_DHCP=y  CONFIG_IP_PNP_BOOTP=y

CONFIG_NFS_FS=y  CONFIG_NFS_V2=y  CONFIG_NFS_V3=y  # CONFIG_NFS_V4 is not set  CONFIG_ROOT_NFS=y  CONFIG_NFS_DISABLE_UDP_SUPPORT=y

CONFIG_SQUASHFS=y  CONFIG_SQUASHFS_ZLIB=y  CONFIG_SQUASHFS_LZO=y  # CONFIG_SQUASHFS_XATTR is not set

CONFIG_SMSC911X=y  CONFIG_MMC_ARMMMCI=y  CONFIG_VFAT_FS=y

Lab 4 boots over NFS and ends by booting the root filesystem from the SD card as a SquashFS image; both are covered by this configuration.

### Cross compiling

$ make -j4

The 6.1 kernel builds with gcc 15.2.0 without warnings (3 min 55 s with -j4 on the test machine). Find the kernel image in the build output and find which .dtb in the Device Tree Source directory matches the board (vexpress-v2p-ca9.dtb):

$ ls -l arch/arm/boot/zImage

$ ls arch/arm/boot/dts | grep vexpress

$ ls -l arch/arm/boot/dts/vexpress-v2p-ca9.dtb

$ file arch/arm/boot/zImage

Expected output:

-rwxrwxr-x 1 longnguyen longnguyen 5500832 Oct 20 17:43 arch/arm/boot/zImage

vexpress-v2m.dtsi

vexpress-v2m-rs1.dtsi

vexpress-v2p-ca15_a7.dtb

vexpress-v2p-ca15_a7.dts

vexpress-v2p-ca15-tc1.dtb

vexpress-v2p-ca15-tc1.dts

vexpress-v2p-ca5s.dtb

vexpress-v2p-ca5s.dts

vexpress-v2p-ca9.dtb

vexpress-v2p-ca9.dts

-rw-rw-r-- 1 longnguyen longnguyen 14081 Oct 20 17:39 arch/arm/boot/dts/vexpress-v2p-ca9.dtb

arch/arm/boot/zImage: Linux kernel ARM boot executable zImage (little-endian)

The toolchain kernel headers (6.1.147) are not newer than this kernel (6.1.190), as required.

### Load and boot the kernel using U-Boot

Copy zImage and vexpress-v2p-ca9.dtb to the directory exported by the TFTP server (root-owned, hence sudo):

$ sudo cp arch/arm/boot/zImage arch/arm/boot/dts/vexpress-v2p-ca9.dtb /srv/tftp/

$ ls -l /srv/tftp

Expected output:

total 5392

-rw-r--r-- 1 root root      27 Oct 20 17:39 textfile.txt

-rw-r--r-- 1 root root   14081 Oct 20 17:43 vexpress-v2p-ca9.dtb

-rwxr-xr-x 1 root root 5500832 Oct 20 17:43 zImage

Start QEMU with the tap network as in Lab 2 (from the bootloader directory, with sudo). In U-Boot, set the kernel command line:

=> setenv bootargs console=ttyAMA0

=> saveenv

Then load the kernel and the device tree and boot:

=> tftp 0x61000000 zImage

=> tftp 0x62000000 vexpress-v2p-ca9.dtb

=> bootz 0x61000000 - 0x62000000

Expected output (the kernel log is shortened):

=> setenv bootargs console=ttyAMA0

=> saveenv

Saving Environment to FAT... OK

=> tftp 0x61000000 zImage

smc911x: detected LAN9118 controller

smc911x: phy initialized

smc911x: MAC 52:54:00:12:34:56

Using ethernet@3,02000000 device

TFTP from server 192.168.0.1; our IP address is 192.168.0.100

Filename 'zImage'.

Load address: 0x61000000

Loading: *#################################################################

#################################################################

#################################################################

#################################################################

#################################################################

##################################################

3.4 MiB/s

done

Bytes transferred = 5500832 (53efa0 hex)

smc911x: MAC 52:54:00:12:34:56

=> tftp 0x62000000 vexpress-v2p-ca9.dtb

smc911x: detected LAN9118 controller

smc911x: phy initialized

smc911x: MAC 52:54:00:12:34:56

Using ethernet@3,02000000 device

TFTP from server 192.168.0.1; our IP address is 192.168.0.100

Filename 'vexpress-v2p-ca9.dtb'.

Load address: 0x62000000

Loading: *#

1.7 MiB/s

done

Bytes transferred = 14081 (3701 hex)

smc911x: MAC 52:54:00:12:34:56

=> bootz 0x61000000 - 0x62000000

Kernel image @ 0x61000000 [ 0x000000 - 0x53efa0 ]

## Flattened Device Tree blob at 62000000

Booting using the fdt blob at 0x62000000

Working FDT set to 62000000

Loading Device Tree to 66b15000, end 66b1b700 ... OK

Working FDT set to 66b15000

Starting kernel ...

Booting Linux on physical CPU 0x0

Linux version 6.1.190 (longnguyen@ubuntu24) (arm-linux-gcc (crosstool-NG 1.28.0) 15.2.0, GNU ld (crosstool-NG 1.28.0) 2.45) #1 SMP Tue Oct 20 17:43:35 UTC 2026

CPU: ARMv7 Processor [410fc090] revision 0 (ARMv7), cr=10c5387d

CPU: PIPT / VIPT nonaliasing data cache, VIPT nonaliasing instruction cache

OF: fdt: Machine model: V2P-CA9

OF: fdt: Ignoring memory block 0x80000000 - 0x80000004

Memory policy: Data cache writeback

Reserved memory: created DMA memory pool at 0x4c000000, size 8 MiB

OF: reserved mem: initialized node vram@4c000000, compatible id shared-dma-pool

cma: Reserved 16 MiB at 0x67000000

Zone ranges:

Normal   [mem 0x0000000060000000-0x0000000067ffffff]

Movable zone start for each node

Early memory node ranges

node   0: [mem 0x0000000060000000-0x0000000067ffffff]

Initmem setup node 0 [mem 0x0000000060000000-0x0000000067ffffff]

CPU: All CPU(s) started in SVC mode.

percpu: Embedded 15 pages/cpu s30996 r8192 d22252 u61440

Built 1 zonelists, mobility grouping on.  Total pages: 32512

Kernel command line: console=ttyAMA0

[...]

Memory: 99936K/131072K available (8192K kernel code, 659K rwdata, 1992K rodata, 1024K init, 181K bss, 14752K reserved, 16384K cma-reserved)

[...]

10009000.uart: ttyAMA0 at MMIO 0x10009000 (irq = 38, base_baud = 0) is a PL011 rev1

printk: console [ttyAMA0] enabled

[...]

mmc0: new SD card at address 4567

[...]

mmcblk0: mmc0:4567 QEMU! 1.00 GiB

[...]

mmcblk0: p1 p2 p3

[...]

/dev/root: Can't open blockdev

VFS: Cannot open root device "(null)" or unknown-block(0,0): error -6

Please append a correct "root=" boot option; here are the available partitions:

1f00          131072 mtdblock0

(driver?)

1f01           32768 mtdblock1

(driver?)

b300         1048576 mmcblk0

driver: mmcblk

b301           65536 mmcblk0p1 ce9f4869-01

b302            8192 mmcblk0p2 ce9f4869-02

b303          973824 mmcblk0p3 ce9f4869-03

Kernel panic - not syncing: VFS: Unable to mount root fs on unknown-block(0,0)

CPU: 0 PID: 1 Comm: swapper/0 Not tainted 6.1.190 #1

Hardware name: ARM-Versatile Express

unwind_backtrace from show_stack+0x10/0x14

show_stack from dump_stack_lvl+0x40/0x4c

dump_stack_lvl from panic+0x104/0x318

panic from mount_block_root+0x1b8/0x25c

mount_block_root from prepare_namespace+0x148/0x184

prepare_namespace from kernel_init+0x18/0x12c

kernel_init from ret_from_fork+0x14/0x28

Exception stack(0x88825fb0 to 0x88825ff8)

5fa0:                                     00000000 00000000 00000000 00000000

5fc0: 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000

5fe0: 00000000 00000000 00000000 00000000 00000013 00000000

---[ end Kernel panic - not syncing: VFS: Unable to mount root fs on unknown-block(0,0) ]---

You should see Linux boot and then panic. This is expected: there is no root filesystem yet (no root= option on the kernel command line). The list of available partitions shows the three partitions of sd.img created in Lab 2. Quit QEMU with Ctrl-a then x.

Restart QEMU and automate the sequence. In U-Boot:

=> setenv bootcmd 'tftp 0x61000000 zImage; tftp 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000'

=> saveenv

=> reset

Expected output after reset (the kernel then boots and panics exactly as above, without any typing):

=> setenv bootcmd 'tftp 0x61000000 zImage; tftp 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000'

=> saveenv

Saving Environment to FAT... OK

=> reset

resetting ...

U-Boot 2023.01 (Oct 20 2026 - 17:37:55 +0000)

DRAM:  128 MiB

WARNING: Caches not enabled

Core:  18 devices, 10 uclasses, devicetree: embed

Flash: 64 MiB

MMC:   mmci@5000: 0

Loading Environment from FAT... OK

In:    serial

Out:   serial

Err:   serial

Net:   eth0: ethernet@3,02000000

Hit any key to stop autoboot:  2  1  0

smc911x: detected LAN9118 controller

smc911x: phy initialized

smc911x: MAC 52:54:00:12:34:56

Using ethernet@3,02000000 device

TFTP from server 192.168.0.1; our IP address is 192.168.0.100

Filename 'zImage'.

Load address: 0x61000000

[...]

Bytes transferred = 5500832 (53efa0 hex)

[...]

Filename 'vexpress-v2p-ca9.dtb'.

Load address: 0x62000000

[...]

Bytes transferred = 14081 (3701 hex)

smc911x: MAC 52:54:00:12:34:56

Kernel image @ 0x61000000 [ 0x000000 - 0x53efa0 ]

## Flattened Device Tree blob at 62000000

Booting using the fdt blob at 0x62000000

Working FDT set to 62000000

Loading Device Tree to 66b15000, end 66b1b700 ... OK

Working FDT set to 66b15000

Starting kernel ...

[...]

Kernel panic - not syncing: VFS: Unable to mount root fs on unknown-block(0,0)

[...]

---[ end Kernel panic - not syncing: VFS: Unable to mount root fs on unknown-block(0,0) ]---

### Exploring the Device Tree

You have just loaded a .dtb file without looking inside it. The Device Tree Blob is the description of the hardware that the bootloader hands to the kernel, which is why bootz takes its address as the third argument. Decompile it back to readable source with the Device Tree Compiler (already installed in Lab 2). Work in a scratch directory:

$ mkdir ~/embedded-linux-qemu-labs/kernel/dt

$ cd ~/embedded-linux-qemu-labs/kernel/dt

$ cp ../linux/arch/arm/boot/dts/vexpress-v2p-ca9.dtb .

$ dtc -I dtb -O dts -o vexpress-v2p-ca9.dts vexpress-v2p-ca9.dtb

dtc prints about eleven warnings and still writes the file; they are normal for this blob (the command exits with status 0):

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/flash@0,00000000: simple-bus unit address format error, expected "0"

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/psram@2,00000000: simple-bus unit address format error, expected "200000000"

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/ethernet@3,02000000: simple-bus unit address format error, expected "302000000"

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/usb@3,03000000: simple-bus unit address format error, expected "303000000"

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/iofpga@7,00000000: simple-bus unit address format error, expected "700000000"

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/fixed-regulator-0: missing or empty reg/ranges property

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/clk24mhz: missing or empty reg/ranges property

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/refclk1mhz: missing or empty reg/ranges property

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/refclk32khz: missing or empty reg/ranges property

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/leds: missing or empty reg/ranges property

vexpress-v2p-ca9.dts: Warning (simple_bus_reg): /bus@40000000/motherboard-bus@40000000/mcc: missing or empty reg/ranges property

The test used DTC 1.7.0; the decompiled file vexpress-v2p-ca9.dts has 723 lines.

Open vexpress-v2p-ca9.dts and find:

- The compatible property of the root node, which identifies the board

- The memory node: compare its address with the RAM start you saw with bdinfo in Lab 2

- The node describing the serial port that the kernel uses for console=ttyAMA0

- The chosen node, if present, and what it is meant to carry

What the test run found (excerpts of the decompiled file):

- Root node. The compatible property lists, from most to least specific, arm,vexpress,v2p-ca9 and arm,vexpress (dtc prints the two strings separated by \0):

/dts-v1/;

/ {

model = "V2P-CA9";

arm,hbi = <0x191>;

arm,vexpress,site = <0x0f>;

compatible = "arm,vexpress,v2p-ca9\0arm,vexpress";

interrupt-parent = <0x01>;

#address-cells = <0x01>;

#size-cells = <0x01>;

- Memory node. It is named memory@60000000 and starts at 0x60000000, the same address as the RAM start shown by bdinfo in Lab 2. The blob describes 1 GiB (0x40000000), while the kernel log above reports only 128 MiB (Memory: 99936K/131072K available, range 0x60000000-0x67ffffff): U-Boot adjusts the memory description to the 128 MiB given to QEMU with -m 128M.

memory@60000000 {

device_type = "memory";

reg = <0x60000000 0x40000000>;

};

- Serial port. The console ttyAMA0 is the node uart@9000 (a PL011 UART, compatible = "arm,pl011"), which the aliases node names serial0. Its register offset 0x9000 is relative to the I/O FPGA node (base 0x10000000), which gives the address in the kernel log line 10009000.uart: ttyAMA0 at MMIO 0x10009000:

uart@9000 {

compatible = "arm,pl011\0arm,primecell";

reg = <0x9000 0x1000>;

interrupts = <0x05>;

clocks = <0x09 0x05>;

clock-names = "uartclk\0apb_pclk";

};

aliases {

serial0 = "/bus@40000000/motherboard-bus@40000000/iofpga@7,00000000/uart@9000";

serial1 = "/bus@40000000/motherboard-bus@40000000/iofpga@7,00000000/uart@a000";

serial2 = "/bus@40000000/motherboard-bus@40000000/iofpga@7,00000000/uart@b000";

serial3 = "/bus@40000000/motherboard-bus@40000000/iofpga@7,00000000/uart@c000";

- chosen node. It is present but empty in the blob. It exists so that the bootloader can pass boot-time data (mainly the kernel command line) to the kernel; U-Boot fills it in before starting the kernel. You will see the filled-in node on the running target in Lab 4 (/sys/firmware/devicetree/base/chosen contains bootargs, name and u-boot,version).

chosen {

};

Then compare your decompiled file with the source in the kernel tree (arch/arm/boot/dts/vexpress-v2p-ca9.dts, 310 lines). The source includes another file, which is already merged in the decompiled output:

12:#include "vexpress-v2m.dtsi"

Differences you can observe: the source reads compatible = "arm,vexpress,v2p-ca9", "arm,vexpress"; and chosen { };, its aliases use labels (serial0 = &v2m_serial0;), and the UART node is defined in vexpress-v2m.dtsi with the label v2m_serial0:

v2m_serial0: uart@9000 {

compatible = "arm,pl011", "arm,primecell";

reg = <0x09000 0x1000>;

interrupts = <5>;

clocks = <&v2m_oscclk2>, <&smbclk>;

clock-names = "uartclk", "apb_pclk";

};

In the decompiled file, labels and &label references are resolved to numeric phandles (for example <0x01>, <0x08 0x05>) and the aliases targets are written as full paths; the file is 723 lines instead of 310 because the included .dtsi content is expanded in it.

## Lab 4 - Tiny embedded system with BusyBox (Session 11)

Objective: Build a tiny but working embedded Linux system whose root filesystem is a directory on your workstation shared over NFS.

After this lab, you will be able to:

- Boot the kernel on a root filesystem exported by NFS

- Create a minimal root filesystem from scratch and install BusyBox in it

- Write a simple startup configuration based on /sbin/init

- Deploy the system to the SD card image and boot it without any network

### Why NFS

While developing a root filesystem you change files often. Reflashing the target every time is impractical. With NFS the target mounts a directory of the workstation, so changes are visible without rebooting.

### Setup

Go to $HOME/embedded-linux-qemu-labs/tinysystem. Reuse the kernel from Lab 3: it already has the NFS root options and CONFIG_DEVTMPFS_MOUNT enabled. Make sure the git identity from the Prerequisites is set (needed for git am).

### Setting up the NFS server

Create a nfsroot directory in the lab directory, install the server (the service starts automatically), and add this line (on one line, no space between the address and the options) to /etc/exports as root:

$ cd ~/embedded-linux-qemu-labs/tinysystem

$ mkdir nfsroot

$ sudo apt install nfs-kernel-server

$ echo "/home/$USER/embedded-linux-qemu-labs/tinysystem/nfsroot 192.168.0.100(rw,no_root_squash,no_subtree_check)" | sudo tee -a /etc/exports

Expected output of the tee command (the line that was appended):

/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot 192.168.0.100(rw,no_root_squash,no_subtree_check)

Apply it and check it:

$ sudo exportfs -r

$ sudo exportfs -v

Expected output of exportfs -v:

/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot

192.168.0.100(sync,wdelay,hide,no_subtree_check,sec=sys,rw,secure,no_root_squash,no_all_squash)

### Booting the system

Start QEMU with the tap network (as in Lab 2). In U-Boot, extend the kernel command line (one line), replace <user> by your user name (longnguyen in the captures), and save:

=> setenv bootargs ${bootargs} root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/<user>/embedded-linux-qemu-labs/tinysystem/nfsroot,nfsvers=3,tcp rw

=> saveenv

=> printenv bootargs

=> boot

Expected output of printenv bootargs:

bootargs=console=ttyAMA0 root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot,nfsvers=3,tcp rw

boot runs the bootcmd saved in Lab 3 (TFTP download of zImage and the .dtb, then bootz). You should see VFS: Mounted root (nfs filesystem). If not, read the console errors and the NFS server log in /var/log/syslog.

The nfsroot directory is still empty, so this first boot is expected to end with a panic. Expected output (the beginning of the kernel log is shortened):

Kernel command line: console=ttyAMA0 root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot,nfsvers=3,tcp rw

[...]

Generic PHY 4e000000.ethernet-ffffffff:01: attached PHY driver (mii_bus:phy_addr=4e000000.ethernet-ffffffff:01, irq=POLL)

smsc911x 4e000000.ethernet eth0: SMSC911x/921x identified at 0x88960000, IRQ: 30

[...]

IP-Config: Guessing netmask 255.255.255.0

IP-Config: Complete:

device=eth0, hwaddr=52:54:00:12:34:56, ipaddr=192.168.0.100, mask=255.255.255.0, gw=255.255.255.255

host=192.168.0.100, domain=, nis-domain=(none)

bootserver=255.255.255.255, rootserver=192.168.0.1, rootpath=

clk: Disabling unused clocks

ALSA device list:

#0: ARM AC'97 Interface PL041 rev0 at 0x10004000, irq 37

VFS: Mounted root (nfs filesystem) on device 0:13.

devtmpfs: error mounting -2

Freeing unused kernel image (initmem) memory: 1024K

Run /sbin/init as init process

Run /etc/init as init process

Run /bin/init as init process

Run /bin/sh as init process

Kernel panic - not syncing: No working init found.  Try passing init= option to kernel. See Linux Documentation/admin-guide/init.rst for guidance.

CPU: 0 PID: 1 Comm: swapper/0 Not tainted 6.1.190 #1

Hardware name: ARM-Versatile Express

unwind_backtrace from show_stack+0x10/0x14

show_stack from dump_stack_lvl+0x40/0x4c

dump_stack_lvl from panic+0x104/0x318

panic from kernel_init+0x128/0x12c

kernel_init from ret_from_fork+0x14/0x28

Exception stack(0x88825fb0 to 0x88825ff8)

5fa0:                                     00000000 00000000 00000000 00000000

5fc0: 00000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000

5fe0: 00000000 00000000 00000000 00000000 00000013 00000000

---[ end Kernel panic - not syncing: No working init found.  Try passing init= option to kernel. See Linux Documentation/admin-guide/init.rst for guidance. ]---

Both messages come from the same boot of the empty root filesystem:

- devtmpfs: error mounting -2 is not fatal: the kernel tries to mount devtmpfs on /dev but the directory does not exist yet. Create a dev directory in nfsroot and restart QEMU:

$ mkdir nfsroot/dev

The message is now devtmpfs: mounted:

VFS: Mounted root (nfs filesystem) on device 0:13.

devtmpfs: mounted

Freeing unused kernel image (initmem) memory: 1024K

Run /sbin/init as init process

Run /etc/init as init process

Run /bin/init as init process

Run /bin/sh as init process

Kernel panic - not syncing: No working init found.  Try passing init= option to kernel. See Linux Documentation/admin-guide/init.rst for guidance.

- Kernel panic - not syncing: No working init found stays: the kernel tried /sbin/init, /etc/init, /bin/init and /bin/sh and found none, because the root filesystem is empty. BusyBox provides the init.

### Root filesystem with BusyBox

$ cd ~/embedded-linux-qemu-labs/tinysystem

$ git clone https://git.busybox.net/busybox

$ cd busybox/

$ git checkout 1_37_stable

$ git describe

Expected output of the last two commands:

Switched to a new branch '1_37_stable'

branch '1_37_stable' set up to track 'origin/1_37_stable'.

1_37_0

Configure BusyBox with the configuration file provided in the lab data/ directory, busybox-1.37.config (it becomes .config). Keep the setting that builds a static BusyBox (CONFIG_STATIC=y): no library is needed on the target. The provided configuration is a minimal applet set (for example there is no head or tail applet).

$ cp ../data/busybox-1.37.config .config

The patch below fixes make menuconfig with newer host compilers (GCC 14 or newer). On the Ubuntu 24.04 host (gcc 13.3) make menuconfig works without it; applying it is harmless (it applied cleanly in the test). It needs the git identity set in the Prerequisites:

$ git am $HOME/embedded-linux-qemu-labs/tinysystem/data/0001-menuconfig-GCC-failing-saying-ncurses-is-not-found.patch

Expected output:

Applying: menuconfig: GCC failing saying ncurses is not found

Make sure CROSS_COMPILE points to the Lab 1 toolchain (the prefix is passed through the environment, so CONFIG_CROSS_COMPILER_PREFIX stays empty), then build with make:

$ export PATH=$HOME/x-tools/arm-training-linux-musleabihf/bin:$PATH

$ export CROSS_COMPILE=arm-linux-

$ make -j4

Expected output (end of the build; it took 8.3 seconds):

Library crypt is not needed, excluding it

Library m is not needed, excluding it

Library rt is not needed, excluding it

Final link with: <none>

Check the result:

$ file busybox

$ ls -l busybox

busybox: ELF 32-bit LSB executable, ARM, EABI5 version 1 (SYSV), statically linked, stripped

-rwxrwxr-x 1 longnguyen longnguyen 365740 Oct 20 17:45 busybox

The installation directory is set by Settings -> Install Options -> Destination path for 'make install' (CONFIG_PREFIX). In the provided busybox-1.37.config it is already "../nfsroot", which is the right path as long as the BusyBox clone is in tinysystem/ next to nfsroot; you can check or change it with make menuconfig. The relevant lines of .config:

CONFIG_STATIC=y

CONFIG_CROSS_COMPILER_PREFIX=""

CONFIG_PREFIX="../nfsroot"

Install BusyBox into nfsroot. CROSS_COMPILE must still be exported in this shell, otherwise the host compiler would be used again:

$ make install

Expected output (end; one symbolic link per applet is created):

../nfsroot//usr/bin/uptime -> ../../bin/busybox

../nfsroot//usr/bin/which -> ../../bin/busybox

../nfsroot//usr/sbin/httpd -> ../../bin/busybox

../nfsroot//usr/sbin/i2cdetect -> ../../bin/busybox

../nfsroot//usr/sbin/seedrng -> ../../bin/busybox

nfsroot now contains bin, dev, sbin and usr (about 388 KB). Restart QEMU (the tap network, autoboot through bootcmd): you should reach a shell prompt. BusyBox init finds no /etc/inittab yet and uses its built-in defaults; press Enter when Please press Enter to activate this console. is shown. Expected output:

VFS: Mounted root (nfs filesystem) on device 0:13.

devtmpfs: mounted

Freeing unused kernel image (initmem) memory: 1024K

Run /sbin/init as init process

starting pid 54, tty '': '/etc/init.d/rcS'

can't run '/etc/init.d/rcS': No such file or directory

Please press Enter to activate this console.

starting pid 55, tty '': '-/bin/sh'

BusyBox v1.37.0 (2026-10-20 17:45:18 UTC) built-in shell (ash)

Enter 'help' for a list of built-in commands.

-/bin/sh: can't access tty; job control turned off

# echo hello from the target

hello from the target

The BusyBox shell prompt is # .

### Virtual filesystems and startup

ps fails because /proc is missing. On the target, create the proc, sys and etc directories in the root filesystem and mount /proc; ps now works (and halt can unmount filesystems cleanly):

# ps

# mkdir /proc /sys /etc

# mount -t proc proc /proc

# ps

# cat /proc/mounts

Expected output (the ps list is shortened):

# ps

PID USER       VSZ STAT COMMAND

ps: can't open '/proc': No such file or directory

# mkdir /proc /sys /etc

# mount -t proc proc /proc

# ps

PID USER       VSZ STAT COMMAND

1 0          524 S    /sbin/init

2 0            0 SW   [kthreadd]

3 0            0 IW<  [rcu_gp]

[...]

55 0          532 S    -/bin/sh

56 0          524 S    /sbin/init

57 0          524 S    /sbin/init

58 0          524 S    /sbin/init

62 0          532 R    ps

# cat /proc/mounts

192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot / nfs rw,relatime,vers=3,rsize=4096,wsize=4096,namlen=255,hard,nolock,proto=tcp,timeo=600,retrans=2,sec=sys,mountaddr=192.168.0.1,mountvers=3,mountproto=tcp,local_lock=all,addr=192.168.0.1 0 0

devtmpfs /dev devtmpfs rw,relatime,size=49968k,nr_inodes=12492,mode=755 0 0

proc /proc proc rw,relatime 0 0

The /proc/mounts output confirms that the root filesystem is the NFS export (vers=3, proto=tcp) and that devtmpfs is mounted on /dev. Because the directories were created on the NFS root, they now exist in nfsroot on the host. At this point /sys is not mounted, so the device tree is not visible yet:

# ls /proc/device-tree/chosen

ls: /proc/device-tree/chosen: No such file or directory

/proc/device-tree is a symbolic link into sysfs. After /sys is mounted (next step) use /sys/firmware/devicetree/base/chosen instead.

Halt the target cleanly with halt (this prints the umount and Requesting system halt messages, then QEMU stays at reboot: System halted; quit QEMU with Ctrl-a then x).

The first user program run by the kernel is /sbin/init, configured by /etc/inittab (see examples/inittab in the BusyBox sources). On the host, create /etc/inittab and a startup script /etc/init.d/rcS declared in it; in the script, mount /proc and /sys. The ttyAMA0::askfirst:/bin/sh line starts the shell on the real terminal device instead of /dev/console, which gets rid of the can't access tty; job control turned off warning:

$ cd ~/embedded-linux-qemu-labs/tinysystem

$ mkdir -p nfsroot/etc/init.d nfsroot/proc nfsroot/sys

$ cat > nfsroot/etc/inittab <<EOF

::sysinit:/etc/init.d/rcS

ttyAMA0::askfirst:/bin/sh

::shutdown:/bin/umount -a -r

EOF

$ cat > nfsroot/etc/init.d/rcS <<EOF

#!/bin/sh

mount -t proc nodev /proc

mount -t sysfs nodev /sys

EOF

$ chmod +x nfsroot/etc/init.d/rcS

The script must be executable. Restart QEMU: the system now starts through init. Expected output (the ps list is shortened; -/bin/sh of the previous boot is now /bin/sh on /dev/ttyAMA0, and there is no job-control warning):

VFS: Mounted root (nfs filesystem) on device 0:13.

devtmpfs: mounted

Freeing unused kernel image (initmem) memory: 1024K

Run /sbin/init as init process

starting pid 53, tty '': '/etc/init.d/rcS'

Please press Enter to activate this console.

starting pid 56, tty '/dev/ttyAMA0': '/bin/sh'

BusyBox v1.37.0 (2026-10-20 17:45:18 UTC) built-in shell (ash)

Enter 'help' for a list of built-in commands.

# ps

PID USER       VSZ STAT COMMAND

1 0          524 S    /sbin/init

2 0            0 SW   [kthreadd]

[...]

56 0          532 S    /bin/sh

57 0          532 R    ps

# cat /proc/mounts

192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot / nfs rw,relatime,vers=3,rsize=4096,wsize=4096,namlen=255,hard,nolock,proto=tcp,timeo=600,retrans=2,sec=sys,mountaddr=192.168.0.1,mountvers=3,mountproto=tcp,local_lock=all,addr=192.168.0.1 0 0

devtmpfs /dev devtmpfs rw,relatime,size=49968k,nr_inodes=12492,mode=755 0 0

nodev /proc proc rw,relatime 0 0

nodev /sys sysfs rw,relatime 0 0

/proc and /sys are now mounted by rcS. You can now read the device tree that U-Boot passed to the kernel (this is the chosen node of Lab 3). The <NUL> markers in the captured output stand for the NUL byte that terminates each string property:

# ls /sys/firmware/devicetree/base/chosen

bootargs        name            u-boot,version

# cat /sys/firmware/devicetree/base/chosen/bootargs

console=ttyAMA0 root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot,nfsvers=3,tcp rw<NUL># cat /sys/firmware/devicetree/base/model

V2P-CA9<NUL># cat /sys/firmware/devicetree/base/compatible

arm,vexpress,v2p-ca9<NUL>arm,vexpress<NUL>#

The chosen node, empty in the .dtb file, now holds the kernel command line (bootargs) and the U-Boot version (u-boot,version).

Check the network and that the NFS root is writable:

# ping -c 2 192.168.0.1

PING 192.168.0.1 (192.168.0.1): 56 data bytes

64 bytes from 192.168.0.1: seq=0 ttl=64 time=6.278 ms

64 bytes from 192.168.0.1: seq=1 ttl=64 time=1.491 ms

--- 192.168.0.1 ping statistics ---

2 packets transmitted, 2 packets received, 0% packet loss

round-trip min/avg/max = 1.491/3.884/6.278 ms

# echo test > /tmp-test; ls -l /tmp-test; rm /tmp-test

-rw-rw-rw-    1         5 Oct 20 17:50 /tmp-test

(The BusyBox ls -l of this configuration prints no user name column.) Halt the target with halt:

# halt

# starting pid 72, tty '': '/bin/umount -a -r'

umount: devtmpfs busy - remounted read-only

The system is going down NOW!

Sent SIGTERM to all processes

Sent SIGKILL to all processes

Requesting system halt

Flash device refused suspend due to active operation (state 20)

Flash device refused suspend due to active operation (state 20)

reboot: System halted

### Deploying the system to the SD card

NFS is convenient while developing, but a real product boots from its own storage: an SD card, or flash. We now move the system you built from the network to the SD card image created in Lab 2, with the kernel and device tree on the FAT partition and the root filesystem on the second partition.

Install the tool that builds SquashFS images, then create an image of your nfsroot directory. SquashFS is a compressed, read-only filesystem: from now on, change files in nfsroot and rebuild the image rather than editing the system on the card.

$ sudo apt install squashfs-tools

$ cd ~/embedded-linux-qemu-labs/tinysystem

$ mksquashfs nfsroot rootfs.sqsh

$ ls -l rootfs.sqsh

Expected output (end of mksquashfs and the ls; mksquashfs keeps the user id 1000 of the files because it was not run as root, which is harmless):

Number of files 3

Number of fragments 1

Number of symbolic links 52

Number of device nodes 0

Number of fifo nodes 0

Number of socket nodes 0

Number of directories 11

Number of hard-links 0

Number of ids (unique uids + gids) 2

Number of uids 2

longnguyen (1000)

root (0)

Number of gids 2

longnguyen (1000)

root (0)

-rw-r--r-- 1 longnguyen longnguyen 208896 Oct 20 17:50 rootfs.sqsh

Check that the image is not larger than the second partition of sd.img: the image is 208896 bytes (204 KiB) and partition 2 is 16384 sectors, that is 8 MiB (8388608 bytes), so it fits with a lot of room. Expose the SD card image again and copy the root filesystem image to its second partition:

$ cd ~/embedded-linux-qemu-labs/bootloader

$ sudo losetup -f --show --partscan sd.img

$ sudo dd if=../tinysystem/rootfs.sqsh of=/dev/loop<x>p2

Expected output (the loop device was /dev/loop10 in the test):

/dev/loop10

408+0 records in

408+0 records out

208896 bytes (209 kB, 204 KiB) copied, 0.00180029 s, 116 MB/s

Mount the first (FAT) partition, copy the kernel image and the device tree to it, then unmount and release the loop device (do not delete uboot.env, it holds your U-Boot environment):

$ sudo mount /dev/loop<x>p1 /mnt

$ ls -l /mnt

$ sudo cp ../kernel/linux/arch/arm/boot/zImage ../kernel/linux/arch/arm/boot/dts/vexpress-v2p-ca9.dtb /mnt

$ ls -l /mnt

$ sudo umount /mnt

$ sudo losetup -d /dev/loop<x>

Expected output of the two ls -l /mnt (before and after the copy):

total 256

-rwxr-xr-x 1 root root 262144 Jan  1  1980 uboot.env

total 5642

-rwxr-xr-x 1 root root  262144 Jan  1  1980 uboot.env

-rwxr-xr-x 1 root root   14081 Oct 20 17:50 vexpress-v2p-ca9.dtb

-rwxr-xr-x 1 root root 5500832 Oct 20 17:50 zImage

Start QEMU with the tap network once more. In U-Boot, point the kernel command line at the second partition of the SD card. The rootwait parameter makes the kernel wait for the SD card to be initialized before it mounts the root filesystem (cards are detected asynchronously). Then load the kernel and the device tree from the FAT partition instead of from TFTP:

=> setenv bootargs console=ttyAMA0 root=/dev/mmcblk0p2 rootwait

=> setenv bootcmd 'fatload mmc 0:1 0x61000000 zImage; fatload mmc 0:1 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000'

=> saveenv

=> printenv bootargs bootcmd

=> fatls mmc 0:1

=> reset

fatload mmc 0:1 loads a file from the first partition of the first MMC controller. No rootfstype= is needed: the kernel detects SquashFS by itself. Expected output:

=> setenv bootargs console=ttyAMA0 root=/dev/mmcblk0p2 rootwait

=> setenv bootcmd 'fatload mmc 0:1 0x61000000 zImage; fatload mmc 0:1 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000'

=> saveenv

Saving Environment to FAT... OK

=> printenv bootargs bootcmd

bootargs=console=ttyAMA0 root=/dev/mmcblk0p2 rootwait

bootcmd=fatload mmc 0:1 0x61000000 zImage; fatload mmc 0:1 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000

=> fatls mmc 0:1

262144   uboot.env

5500832   zImage

14081   vexpress-v2p-ca9.dtb

3 file(s), 0 dir(s)

=> reset

resetting ...

U-Boot 2023.01 (Oct 20 2026 - 17:37:55 +0000)

[...]

Hit any key to stop autoboot:  2  1  0

5500832 bytes read in 4123 ms (1.3 MiB/s)

14081 bytes read in 19 ms (723.6 KiB/s)

Kernel image @ 0x61000000 [ 0x000000 - 0x53efa0 ]

## Flattened Device Tree blob at 62000000

Booting using the fdt blob at 0x62000000

Working FDT set to 62000000

Loading Device Tree to 66b15000, end 66b1b700 ... OK

Working FDT set to 66b15000

Starting kernel ...

[...]

Kernel command line: console=ttyAMA0 root=/dev/mmcblk0p2 rootwait

[...]

VFS: Mounted root (squashfs filesystem) readonly on device 179:2.

devtmpfs: mounted

Freeing unused kernel image (initmem) memory: 1024K

Run /sbin/init as init process

starting pid 53, tty '': '/etc/init.d/rcS'

Please press Enter to activate this console.

starting pid 56, tty '/dev/ttyAMA0': '/bin/sh'

BusyBox v1.37.0 (2026-10-20 17:45:18 UTC) built-in shell (ash)

Enter 'help' for a list of built-in commands.

#

#### Boot without any network

To prove the system no longer depends on the network, stop QEMU (halt, then Ctrl-a then x) and start it again without the tap options (-net tap and -net nic), and without sudo:

$ cd ~/embedded-linux-qemu-labs/bootloader

$ qemu-system-arm -M vexpress-a9 -m 128M -nographic -kernel u-boot/u-boot -sd sd.img

The system must boot to the shell. Expected output (the kernel log is shortened; fatload now reads the files from the SD card, there is no TFTP step):

U-Boot 2023.01 (Oct 20 2026 - 17:37:55 +0000)

DRAM:  128 MiB

WARNING: Caches not enabled

Core:  18 devices, 10 uclasses, devicetree: embed

Flash: 64 MiB

MMC:   mmci@5000: 0

Loading Environment from FAT... OK

In:    serial

Out:   serial

Err:   serial

Net:   eth0: ethernet@3,02000000

Hit any key to stop autoboot:  2  1  0

5500832 bytes read in 4031 ms (1.3 MiB/s)

14081 bytes read in 21 ms (654.3 KiB/s)

Kernel image @ 0x61000000 [ 0x000000 - 0x53efa0 ]

## Flattened Device Tree blob at 62000000

Booting using the fdt blob at 0x62000000

Working FDT set to 62000000

Loading Device Tree to 66b15000, end 66b1b700 ... OK

Working FDT set to 66b15000

Starting kernel ...

Booting Linux on physical CPU 0x0

Linux version 6.1.190 (longnguyen@ubuntu24) (arm-linux-gcc (crosstool-NG 1.28.0) 15.2.0, GNU ld (crosstool-NG 1.28.0) 2.45) #1 SMP Tue Oct 20 17:43:35 UTC 2026

[...]

Kernel command line: console=ttyAMA0 root=/dev/mmcblk0p2 rootwait

[...]

VFS: Mounted root (squashfs filesystem) readonly on device 179:2.

devtmpfs: mounted

Freeing unused kernel image (initmem) memory: 1024K

Run /sbin/init as init process

starting pid 53, tty '': '/etc/init.d/rcS'

Please press Enter to activate this console.

starting pid 56, tty '/dev/ttyAMA0': '/bin/sh'

BusyBox v1.37.0 (2026-10-20 17:45:18 UTC) built-in shell (ash)

Enter 'help' for a list of built-in commands.

#

Verify the root filesystem. Note that /proc/mounts shows the root device under the generic name /dev/root; to tie it to /dev/mmcblk0p2, compare the major and minor numbers (179:2), as the commands below do:

# cat /proc/mounts

# dmesg | grep -E "VFS|mmcblk0"

# ls -l /dev/mmcblk0p2

# cat /proc/partitions

# touch /x

Expected output:

# cat /proc/mounts

/dev/root / squashfs ro,relatime,errors=continue 0 0

devtmpfs /dev devtmpfs rw,relatime,size=49968k,nr_inodes=12492,mode=755 0 0

nodev /proc proc rw,relatime 0 0

nodev /sys sysfs rw,relatime 0 0

# dmesg | grep -E "VFS|mmcblk0"

Kernel command line: console=ttyAMA0 root=/dev/mmcblk0p2 rootwait

mmcblk0: mmc0:4567 QEMU! 1.00 GiB

mmcblk0: p1 p2 p3

VFS: Mounted root (squashfs filesystem) readonly on device 179:2.

# ls -l /dev/mmcblk0p2

brw-------    1  179,   2 Oct 20 17:51 /dev/mmcblk0p2

# cat /proc/partitions

major minor  #blocks  name

31        0     131072 mtdblock0

31        1      32768 mtdblock1

179        0    1048576 mmcblk0

179        1      65536 mmcblk0p1

179        2       8192 mmcblk0p2

179        3     973824 mmcblk0p3

# touch /x

touch: /x: Read-only file system

This proves that the root filesystem is a read-only SquashFS (squashfs ro) mounted from the device with major:minor 179:2, that is /dev/mmcblk0p2 (8192 blocks of 1 KiB = 8 MiB, the second partition of sd.img), and that writes are refused.

Check that the network is really absent. QEMU still creates a default user-mode network device, so eth0 exists but it is down and not configured:

# ifconfig -a

eth0      Link encap:Ethernet  HWaddr 52:54:00:12:34:56

BROADCAST MULTICAST  MTU:1500  Metric:1

RX packets:0 errors:0 dropped:0 overruns:0 frame:0

TX packets:0 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:0 (0.0 B)  TX bytes:0 (0.0 B)

Interrupt:30

lo        Link encap:Local Loopback

LOOPBACK  MTU:65536  Metric:1

RX packets:0 errors:0 dropped:0 overruns:0 frame:0

TX packets:0 errors:0 dropped:0 overruns:0 carrier:0

collisions:0 txqueuelen:1000

RX bytes:0 (0.0 B)  TX bytes:0 (0.0 B)

# ping -c 1 192.168.0.1

PING 192.168.0.1 (192.168.0.1): 56 data bytes

ping: sendto: Network unreachable

The ipaddr and serverip variables are still stored in the U-Boot environment on the SD card; this is harmless. Halt the target with halt.

## Lab 5 - System build with Buildroot (Session 11)

Objective: Discover how a build system works, with the example of Buildroot, and build a complete Linux system, including the Linux kernel.

### Goals

Labs 3 and 4 built the kernel and the root filesystem by hand. Buildroot automates the same work. We configure it to reuse your Lab 1 toolchain, build the kernel and a BusyBox-based root filesystem, and boot the result.

### Setup

Go to $HOME/embedded-linux-qemu-labs/buildroot.

### Get Buildroot and explore the source code

$ cd ~/embedded-linux-qemu-labs/buildroot

$ git clone https://gitlab.com/buildroot.org/buildroot.git

$ cd buildroot

$ git checkout 2026.02.3

Note: the Buildroot release must offer a "15.x" choice for External toolchain gcc version (BR2_TOOLCHAIN_EXTERNAL_GCC_15), because Buildroot checks the selected gcc series against the real compiler and the Lab 1 toolchain uses gcc 15.2.0. This guide therefore uses release 2026.02.3 (the current long term support release at the time of the test, and the only release built in the test). The kernel headers choice is labelled "6.1.x" in this release.

Browse the main directories:

- boot: bootloader build rules (GRUB, U-Boot, Barebox, ...)

- board: board-specific configurations and root filesystem overlays

- configs: predefined configurations (like a kernel defconfig)

- docs: Buildroot documentation

- fs: code to generate the filesystem image formats

- linux: Linux kernel build rules

- package: build rules, patches and options for user-space applications and libraries

- system: root filesystem skeleton and device tables

- toolchain: rules to generate a cross-compiling toolchain

- Makefile: the main Makefile; everything works through Makefiles

### Configure Buildroot

We want Buildroot to: generate an ARM system, use an existing external toolchain instead of building one, build the Linux kernel, include BusyBox, and put the target filesystem in a tarball. Run make menuconfig and set the options below (the .config symbol that each choice produces is given in parentheses):

- Target options: Target Architecture ARM (little endian) (BR2_arm=y); Target Architecture Variant cortex-A9 (BR2_cortex_a9=y); Enable NEON SIMD extension support (BR2_ARM_ENABLE_NEON=y); Enable VFP extension support (BR2_ARM_ENABLE_VFP=y); Target ABI EABIhf (BR2_ARM_EABIHF=y); Floating point strategy VFPv3-D16 (BR2_ARM_FPU_VFPV3D16=y)

- Toolchain: Toolchain type External toolchain (BR2_TOOLCHAIN_EXTERNAL=y); Toolchain Custom toolchain (BR2_TOOLCHAIN_EXTERNAL_CUSTOM=y); Toolchain path /home/<user>/x-tools/arm-training-linux-musleabihf (BR2_TOOLCHAIN_EXTERNAL_PATH); External toolchain gcc version 15.x (BR2_TOOLCHAIN_EXTERNAL_GCC_15=y); External toolchain kernel headers series 6.1.x (BR2_TOOLCHAIN_EXTERNAL_HEADERS_6_1=y); External toolchain C library musl (experimental) (BR2_TOOLCHAIN_EXTERNAL_CUSTOM_MUSL=y); select Toolchain has SSP support? (BR2_TOOLCHAIN_EXTERNAL_HAS_SSP=y) and Toolchain has C++ support? (BR2_TOOLCHAIN_EXTERNAL_CXX=y) (Buildroot checks these anyway). The default toolchain prefix $(ARCH)-linux (arm-linux) matches the alias set in Lab 1, so it needs no change.

- Kernel: Enable Linux Kernel (BR2_LINUX_KERNEL=y); Kernel version Custom version 6.1.190 (BR2_LINUX_KERNEL_CUSTOM_VERSION=y, BR2_LINUX_KERNEL_CUSTOM_VERSION_VALUE="6.1.190"). In this release "Latest version" is 6.19.14, so the custom version keeps the 6.1 kernel of Lab 3 that the toolchain headers (6.1.x) and the other labs assume. Kernel configuration Using an in-tree defconfig file, Defconfig name vexpress (BR2_LINUX_KERNEL_USE_DEFCONFIG=y, BR2_LINUX_KERNEL_DEFCONFIG="vexpress"); select Build a Device Tree Blob (DTB) (BR2_LINUX_KERNEL_DTS_SUPPORT=y); In-tree Device Tree Source file names vexpress-v2p-ca9 (BR2_LINUX_KERNEL_INTREE_DTS_NAME="vexpress-v2p-ca9")

- Target packages: keep BusyBox (default version) and its proposed configuration (BR2_PACKAGE_BUSYBOX=y)

- Filesystem images: select tar the root filesystem (BR2_TARGET_ROOTFS_TAR=y)

Exit the interface; the configuration is saved in .config. The key lines of .config are:

BR2_arm=y

BR2_cortex_a9=y

BR2_ARM_ENABLE_NEON=y

BR2_ARM_ENABLE_VFP=y

BR2_ARM_EABIHF=y

BR2_ARM_FPU_VFPV3D16=y

BR2_TOOLCHAIN_EXTERNAL=y

BR2_TOOLCHAIN_EXTERNAL_CUSTOM=y

BR2_TOOLCHAIN_EXTERNAL_PREINSTALLED=y

BR2_TOOLCHAIN_EXTERNAL_PATH="/home/longnguyen/x-tools/arm-training-linux-musleabihf"

BR2_TOOLCHAIN_EXTERNAL_MUSL=y

BR2_TOOLCHAIN_EXTERNAL_PREFIX="$(ARCH)-linux"

BR2_TOOLCHAIN_EXTERNAL_CUSTOM_PREFIX="$(ARCH)-linux"

BR2_TOOLCHAIN_EXTERNAL_GCC_15=y

BR2_TOOLCHAIN_EXTERNAL_HEADERS_6_1=y

BR2_TOOLCHAIN_EXTERNAL_CUSTOM_MUSL=y

BR2_TOOLCHAIN_EXTERNAL_HAS_SSP=y

BR2_TOOLCHAIN_EXTERNAL_CXX=y

BR2_LINUX_KERNEL=y

BR2_LINUX_KERNEL_CUSTOM_VERSION=y

BR2_LINUX_KERNEL_CUSTOM_VERSION_VALUE="6.1.190"

BR2_LINUX_KERNEL_VERSION="6.1.190"

BR2_LINUX_KERNEL_USE_DEFCONFIG=y

BR2_LINUX_KERNEL_DEFCONFIG="vexpress"

BR2_LINUX_KERNEL_ZIMAGE=y

BR2_LINUX_KERNEL_GZIP=y

BR2_LINUX_KERNEL_DTS_SUPPORT=y

BR2_LINUX_KERNEL_INTREE_DTS_NAME="vexpress-v2p-ca9"

BR2_PACKAGE_BUSYBOX=y

BR2_TARGET_ROOTFS_TAR=y

(Some lines of the real .config that are empty values, such as BR2_LINUX_KERNEL_PATCH="", are omitted here.)

### Generate the system

Run make from a shell where ARCH and CROSS_COMPILE are not exported:

$ make

Buildroot creates an environment around the external toolchain, then downloads, extracts, configures, compiles and installs each component. Start this build as early as you can: it takes a while (8 min 13 s on the 4 vCPU test machine, with about 154 MB downloaded into dl/). The BusyBox 1.37.0 syslogd compiles with -Wdiscarded-qualifiers warnings under gcc 15; they are warnings, not errors.

When it finishes, check the result:

$ ls -l output/images

$ ls output

$ du -sh output dl

Expected output:

total 9720

-rw-r--r-- 1 longnguyen longnguyen 4433920 Oct 20 18:04 rootfs.tar

-rwxr-xr-x 1 longnguyen longnguyen   14081 Oct 20 18:04 vexpress-v2p-ca9.dtb

-rw-r--r-- 1 longnguyen longnguyen 5500832 Oct 20 18:04 zImage

build

host

images

staging

target

output takes 2.7 GB and dl 154 MB. Everything is built under output/:

- build: where each component is extracted and compiled

- host: tools installed on the host to build the target packages (for example pkg-config, genext2fs, makedevs, fakeroot)

- images: the final images: here rootfs.tar plus the compressed kernel and the device tree binary

- staging: the build space of the target system (libraries, headers, documentation, and the C library copied from the toolchain)

- target: the target root filesystem with stripped binaries; it cannot be used as is because the device files are missing (they cannot be created without root, and Buildroot never runs anything as root)

zImage (5500832 bytes) and the .dtb (14081 bytes) have the same size as the Lab 3 files: same defconfig, kernel version and compiler.

### Run the generated system

Go back to $HOME/embedded-linux-qemu-labs/buildroot. Create a new nfsroot directory and extract the root filesystem into it. The tarball contains no device nodes (the kernel mounts devtmpfs), so extracting it as a regular user works:

$ cd ~/embedded-linux-qemu-labs/buildroot

$ mkdir nfsroot && cd nfsroot

$ tar xvf ../buildroot/output/images/rootfs.tar

Expected output (end of the listing; tar exits with status 0):

./var/spool

./var/tmp

./var/www/

Files extracted this way belong to your user (uid 1000) and the setuid bit of busybox is not preserved; boot and login are not affected.

Add this new nfsroot to /etc/exports (and run sudo exportfs -r):

$ echo "/home/$USER/embedded-linux-qemu-labs/buildroot/nfsroot 192.168.0.100(rw,no_root_squash,no_subtree_check)" | sudo tee -a /etc/exports

$ sudo exportfs -r

$ sudo exportfs -v

/etc/exports now contains both lines, and exportfs -v lists both directories:

/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot 192.168.0.100(rw,no_root_squash,no_subtree_check)

/home/longnguyen/embedded-linux-qemu-labs/buildroot/nfsroot 192.168.0.100(rw,no_root_squash,no_subtree_check)

/home/longnguyen/embedded-linux-qemu-labs/tinysystem/nfsroot

192.168.0.100(sync,wdelay,hide,no_subtree_check,sec=sys,rw,secure,no_root_squash,no_all_squash)

/home/longnguyen/embedded-linux-qemu-labs/buildroot/nfsroot

192.168.0.100(sync,wdelay,hide,no_subtree_check,sec=sys,rw,secure,no_root_squash,no_all_squash)

Update the kernel and device tree served over TFTP with the ones from output/images/ (the Buildroot kernel replaces the Lab 3 files):

$ cd ~/embedded-linux-qemu-labs/buildroot

$ sudo cp buildroot/output/images/zImage buildroot/output/images/vexpress-v2p-ca9.dtb /srv/tftp/

$ ls -l /srv/tftp

total 5392

-rw-r--r-- 1 root root      27 Oct 20 17:39 textfile.txt

-rw-r--r-- 1 root root   14081 Oct 20 18:19 vexpress-v2p-ca9.dtb

-rwxr-xr-x 1 root root 5500832 Oct 20 18:19 zImage

Start QEMU with the tap network (as in Lab 2). Lab 4 left U-Boot's saved bootcmd (load from the SD card) and bootargs (root=/dev/mmcblk0p2) in place, so set both back: the boot command must use TFTP again, and the kernel command line must point to the new nfsroot path:

=> setenv bootcmd 'tftp 0x61000000 zImage; tftp 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000'

=> setenv bootargs console=ttyAMA0 root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/<user>/embedded-linux-qemu-labs/buildroot/nfsroot,nfsvers=3,tcp rw

=> saveenv

=> printenv bootcmd bootargs

=> boot

Boot the board and log in as root (no password): you reach a shell. Expected output (the kernel log is shortened; the commands after buildroot login: were typed on the target):

=> setenv bootcmd 'tftp 0x61000000 zImage; tftp 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000'

=> setenv bootargs console=ttyAMA0 root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/buildroot/nfsroot,nfsvers=3,tcp rw

=> saveenv

Saving Environment to FAT... OK

=> printenv bootcmd bootargs

bootcmd=tftp 0x61000000 zImage; tftp 0x62000000 vexpress-v2p-ca9.dtb; bootz 0x61000000 - 0x62000000

bootargs=console=ttyAMA0 root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/buildroot/nfsroot,nfsvers=3,tcp rw

=> boot

smc911x: detected LAN9118 controller

smc911x: phy initialized

smc911x: MAC 52:54:00:12:34:56

Using ethernet@3,02000000 device

TFTP from server 192.168.0.1; our IP address is 192.168.0.100

Filename 'zImage'.

Load address: 0x61000000

Loading: *#################################################################

[...]

Bytes transferred = 5500832 (53efa0 hex)

[...]

Bytes transferred = 14081 (3701 hex)

smc911x: MAC 52:54:00:12:34:56

Kernel image @ 0x61000000 [ 0x000000 - 0x53efa0 ]

## Flattened Device Tree blob at 62000000

Booting using the fdt blob at 0x62000000

Working FDT set to 62000000

Loading Device Tree to 66b15000, end 66b1b700 ... OK

Working FDT set to 66b15000

Starting kernel ...

Booting Linux on physical CPU 0x0

Linux version 6.1.190 (longnguyen@ubuntu24) (arm-linux-gcc (crosstool-NG 1.28.0) 15.2.0, GNU ld (crosstool-NG 1.28.0) 2.45) #1 SMP Tue Oct 20 18:04:32 UTC 2026

[...]

Kernel command line: console=ttyAMA0 root=/dev/nfs ip=192.168.0.100 nfsroot=192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/buildroot/nfsroot,nfsvers=3,tcp rw

[...]

IP-Config: Guessing netmask 255.255.255.0

IP-Config: Complete:

device=eth0, hwaddr=52:54:00:12:34:56, ipaddr=192.168.0.100, mask=255.255.255.0, gw=255.255.255.255

host=192.168.0.100, domain=, nis-domain=(none)

bootserver=255.255.255.255, rootserver=192.168.0.1, rootpath=

clk: Disabling unused clocks

ALSA device list:

#0: ARM AC'97 Interface PL041 rev0 at 0x10004000, irq 37

VFS: Mounted root (nfs filesystem) on device 0:13.

devtmpfs: mounted

Freeing unused kernel image (initmem) memory: 1024K

Run /sbin/init as init process

Saving 256 bits of non-creditable seed for next boot

Starting syslogd: OK

Starting klogd: OK

Running sysctl: OK

Starting network: ip: RTNETLINK answers: File exists

FAIL

Starting crond: OK

Welcome to Buildroot

buildroot login: root

# uname -a

Linux buildroot 6.1.190 #1 SMP Tue Oct 20 18:04:32 UTC 2026 armv7l GNU/Linux

# cat /etc/os-release

NAME=Buildroot

VERSION=2026.02.3

ID=buildroot

VERSION_ID=2026.02.3

PRETTY_NAME="Buildroot 2026.02.3"

# cat /proc/mounts

192.168.0.1:/home/longnguyen/embedded-linux-qemu-labs/buildroot/nfsroot / nfs rw,relatime,vers=3,rsize=4096,wsize=4096,namlen=255,hard,nolock,proto=tcp,timeo=600,retrans=2,sec=sys,mountaddr=192.168.0.1,mountvers=3,mountproto=tcp,local_lock=all,addr=192.168.0.1 0 0

devtmpfs /dev devtmpfs rw,relatime,size=49968k,nr_inodes=12492,mode=755 0 0

proc /proc proc rw,relatime 0 0

devpts /dev/pts devpts rw,relatime,gid=5,mode=620,ptmxmode=666 0 0

tmpfs /dev/shm tmpfs rw,relatime 0 0

tmpfs /tmp tmpfs rw,relatime 0 0

tmpfs /run tmpfs rw,nosuid,nodev,relatime,mode=755 0 0

sysfs /sys sysfs rw,relatime 0 0

# ps

PID   USER     COMMAND

1 root     init

2 root     [kthreadd]

[...]

72 root     /sbin/syslogd -n

76 root     /sbin/klogd -n

97 root     /usr/sbin/crond -f

98 root     -sh

103 root     ps

# ifconfig

eth0      Link encap:Ethernet  HWaddr 52:54:00:12:34:56

inet addr:192.168.0.100  Bcast:192.168.0.255  Mask:255.255.255.0

UP BROADCAST RUNNING MULTICAST  MTU:1500  Metric:1

[...]

# ls /etc/init.d

S01seedrng  S02klogd    S11modules  S50crond    rcS

S01syslogd  S02sysctl   S40network  rcK

Starting network: ip: RTNETLINK answers: File exists / FAIL is harmless: Buildroot's network start script tries to configure eth0, which the kernel has already configured from the ip=192.168.0.100 option; eth0 is up with 192.168.0.100 as the ifconfig output shows. Compare the Buildroot system with the one you built by hand in Lab 4: /etc/inittab (the extracted etc/inittab of the Buildroot tarball starts the scripts of /etc/init.d and a getty on the console), the S* init scripts, and the many more BusyBox applets (ps shows a USER name instead of a numeric id).

Power off the target with poweroff:

# poweroff

# Stopping crond: stopped /usr/sbin/crond (pid 97)

OK

Stopping network: ifdown: interface lo not configured

OK

Stopping klogd: stopped /sbin/klogd (pid 76)

OK

Stopping syslogd: stopped /sbin/syslogd (pid 72)

OK

Seeding 256 bits without crediting

Saving 256 bits of non-creditable seed for next boot

umount: devtmpfs busy - remounted read-only

The system is going down NOW!

Sent SIGTERM to all processes

Sent SIGKILL to all processes

Requesting system poweroff